Topic 1a

- Basic functions of a computer:

  - Input – get data from input devices (keyboard, mouse, microphone etc.)

  - Process – processing unit (CPU, graphic processor etc.) process the data

  - Output – output data to output devices (display monitor, speaker/headphone, printer etc.)

- Storage components of computer:

  - Short-term storage

    - Main memory (RAM: Random Access Memory)

    - Volatile; when power is turned off, RAM’s content will be gone

    - Stores program code/instructions and data temporary during execution of programs, sometimes termed as working storage

    - CPU can only access program code/instructions and data that are in main memory (and not in the permanent storage such as hard disk)

    - Virtual memory – utilising part of hard disk storage to supplement the main memory, so that more programs/data can be loaded (but it does make programs run faster)

  - Long-term storage

    - Non-volatile; maintain contents stored even when there is no power

    - Used to store program and data permanently

    - Examples: hard disk, USB thumbdrive, CD/DVD

- Types of RAM: Static (SRAM) vs Dynamic (DRAM)

Static RAM (SRAM)Very Fast

  - But Costly

  - Use in Cache Memory

  - Do not need constant refresh of power to retain data/instructions stored in it

Dynamic RAM (DRAM)

  - Cost Effective

  - But Slow

  - Use in main memory

  - Need constant refresh of power to retain data/instructions stored in it

How CPU (ALU and Control Unit), Memory (RAM) and Input/Output components are connected?

- By system bus – a collections of wires printed on the motherboard.

- System Bus (Interconnection Unit) comprises of:

  - Data bus (carries data to be stored or read) between RAM and CPU, and between RAM and input/output devices

  - Address bus (carries the address of location to where data should be stored or read)

  - Control bus (carries control signal like to read or write)

- CPU that contains multiple physical processors called cores

  - Allows computers to do multi-taskings

  - Executing multiple instructions simultaneously

- How CPU gives attention to devices?

  - Polling

    - CPU keeps checking on I/O devices at regular interval to see if needs the service of the CPU

  - Interrupt

    - I/O device interrupts the CPU to request for the service of the CPU

- How to make your program runs faster?

  - Get a faster CPU, with multiple cores and larger cache memory

  - Get more and faster RAM

  - Use SSD (Solid State Disk), these disks have no moving parts and hence faster than normal disk drives

- What is BIOS?

  - Basic input/output system

  - Set of instructions located in a ROM (read only memory) chip on the motherboard

  - Tells the CPU to perform certain tasks when power is first applied to the computer

  - This includes instructions to perform a power-on self-test (POST)

  - BIOS is a type of firmware (program instructions stored in hardware such as ROM)

- What is CMOS?

- Complementary Metal Oxide Semiconductor – a type of RAM

- Need constant power, provided by a cell button battery on the motherboard, to retain data stored in it

- Stores settings for BIOS such as date and time, boot sequence (priority) i.e. boot from which hard disk and password for BIOS

- UEFI (Unified Extensible Firmware Interface) – a newer replacement for BIOS (much easier to interact with than BIOS, as it has a graphical user interface

- Computer boot procedure:

- Power is applied to the motherboard

- The CPU starts

- The CPU carries out the BIOS startup routines,  including the POST (Power-on self-test)

- Boot devices, as specified in the BIOS configuration, are searched for an OS (Windows, Linux etc.) to be loaded

- The OS (Windows, Linux etc.) is loaded into RAM

- OS (Windows, Linux etc.) is started and you can then use your computer.

How CPU works - Scott CPU?

https://www.youtube.com/watch?v=cNN_tTXABUA

CPU Cache Explained - What is Cache Memory?

https://www.youtube.com/watch?v=yi0FhRqDJfo

RAM Explained - Random Access Memory

https://www.youtube.com/watch?v=PVad0c2cljo

BIOS, CMOS, UEFI - What's the difference?

https://www.youtube.com/watch?v=LGz0Io_dh_I

32-bit vs 64-bit CPU

https://www.youtube.com/watch?v=Wu2A4fpFzgs

How do hard drive works?

https://www.youtube.com/watch?v=wI0upu9eVcw

Hyper-Threading (Multi-Core, Multi-Threading)

https://www.youtube.com/watch?v=lrT9Bl0MCXQ&t=165s